Supervisor: Dr. KANOUNI Fares Senior Researcher, Class A

Photonic Crystals Team, Research Unit in Optics and Photonics

Research Unit in Optics and Photonics UROP Center for Development of Advanced Technologies (UROP-CDTA)

19000, Setif, El-Bez, Algeria

# i	Abstract

This work describes the complete development of a two-dimensional phononic crystal (2D PnC) sensor for liquid mixture characterization. This research is divided into three distinct sections. The first part of this work involves creating a COMSOL Multiphysics model of the phononic-fluidic sensor to study acoustic wave propagation and sensor response through detailed simulations. The second section describ the optimization process followed by fabrication of the 2D phononic crystal, which features a square lattice of air holes inside a stainless steel matrix. The developed structure served for identifying different unknown liquid mixtures, including acetone–water, nickel chlo- ride–water, and sodium chloride–water solutions. In addition, the experimental measurement of the acoustic transmission spectra led to the development of calibration curves for each mixture, which enables concentration identification. Moreover, this study evaluates the sensor performance through measurements of sensitivity and quality factor (Q-factor). The third section of this research involves creating a graphical user interface (GUI) to support real-time data acquisition and analysis. The interface underwent testing, which confirmed its reliable operational performance.

# Résumé

Ce travail décrit le développement complet d’un capteur à cristaux phononiques bidimension- nels (2D PnC) pour la caractérisation des mélanges liquides complexes. Cette recherche se divise en trois sections principales. La première partie inclut la création d’un modèle en utilisant COM- SOL Multiphysics pour ce capteur acoustique afin d’étudier la propagation des ondes sonores et la réponse du capteur à travers une simulation détaillée. La deuxième section décrit le processus d’optimisation qui a suivi la fabrication de la structure phononique bidimensionnelle, caractérisée par un réseau carré de trous à l’intérieur d’une matrice en acier inoxydable. Ce capteur a été utilisé pour identifier des concentration des mélanges liquides inconnus, y compris le mélange acétone- eau, le mélange chlorure de nickel-eau et le mélange chlorure de sodium-eau. De plus, les mesures expérimentales du spectre de transmission acoustique ont conduit au développement de courbes d’étalon- nage pour chaque mélange, permettant ainsi de déterminer la concentration. Cette étude a également montré les performances du capteur à travers des mesures de sensibilité et du facteur de qualité (Q-factor). La troisième section a inclus la création d’une interface utilisateur graphique (GUI) pour soutenir la collecter et analyser les données en temps réel. L’interface a été testée, confirmant ainsi ses performances opérationnelles fiables.

# ﺍﻟﻤﻠﺨص

ﻳﺼﻒ ﻫﺬﺍ ﺍﻟﻌﻤﻞ ﺍﻟﺘﻄﻮﻳﺮ ﺍﻟﺸﺎﻣﻞ ﻟﺠﻬﺎﺯ ﺍﺳﺘﺸﻌﺎﺭ ﺑﻠﻮﺭﺓ ﺻﻮﺗﻴﺔ ﺛﻨﺎﺋﻴﺔ ﺍﻷﺑﻌﺎﺩ ﻟﺘﻮﺻﻴﻒ ﺍﻟﺨﻠﻄﺎﺕ ﺍﻟﺴﺎﺋﻠﺔ. ﻳﻨﻘﺴﻢ ﻫﺬﺍ

ﺍﻟﺒﺤﺚ ﺇﻟﻰ ﺛﻼﺛﺔ ﺃﻗﺴﺎﻡ ﺭﺋﻴﺴﻴﺔ. ﺍﻟﺠﺰء ﺍﻷﻭﻝ ﺗﻀﻤﻦ ﺇﻧﺸﺎء ﻧﻤﻮﺫﺝ ﺑﺎﺳﺘﺨﺪﺍﻡ  Multiphysics COMSOL  ﻟﻬﺬﺍ ﺍﻟﺠﻬﺎﺯ

ﺍﻻﺳﺘﺸﻌﺎﺭ ﺍﻟﺼﻮﺗﻲ ﻟﺪﺭﺍﺳﺔ ﺍﻧﺘﺸﺎﺭ ﺍﻟﻤﻮﺟﺎﺕ ﺍﻟﺼﻮﺗﻴﺔ ﻭﺍﺳﺘﺠﺎﺑﺔ ﺍﻟﻤﺴﺘﺸﻌﺮ ﻣﻦ ﺧﻼﻝ ﻣﺤﺎﻛﺎﺓ ﻣﻔﺼﻠﺔ. ﺍﻟﻘﺴﻢ ﺍﻟﺜﺎﻧﻲ ﻳﺼﻒ ﻋﻤﻠﻴﺔ ﺍﻟﺘﺤﺴﻴﻦ ﺍﻟﺘﻲ ﺗﻠﺘﻬﺎ ﺗﺼﻨﻴﻊ ﺍﻟﺒﻠﻮﺭﺓ ﺍﻟﺼﻮﺗﻴﺔ ﺛﻨﺎﺋﻴﺔ ﺍﻷﺑﻌﺎﺩ، ﻭﺍﻟﺘﻲ ﺗﺘﻤﻴﺰ ﺑﺸﺒﻜﺔ ﻣﺮﺑﻌﺔ ﻣﻦ ﺍﻟﺜﻘﻮﺏ ﺍﻟﻬﻮﺍﺋﻴﺔ ﺩﺍﺧﻞ ﻣﺼﻔﻮﻓﺔ ﻣﻦ ﺍﻟﻔﻮﻻﺫ ﺍﻟﻤﻘﺎﻭﻡ ﻟﻠﺼﺪﺃ. ﺍﺳﺘﺨﺪﻡ ﻫﺬﺍ ﺍﻟﻤﺴﺘﺸﻌﺮ ﻟﺘﺤﺪﻳﺪ ﻣﺨﺘﻠﻒ ﺍﻟﺨﻠﻄﺎﺕ ﺍﻟﺴﺎﺋﻠﺔ ﻏﻴﺮ ﺍﻟﻤﻌﺮﻭﻓﺔ، ﺑﻤﺎ ﻓﻲ ﺫﻟﻚ ﺧﻠﻴﻂ ﺍﻷﺳﻴﺘﻮﻥ ﻭﺍﻟﻤﺎء، ﻭﺧﻠﻴﻂ

ﻛﻠﻮﺭﻳﺪ ﺍﻟﻨﻴﻜﻞ ﻭﺍﻟﻤﺎء، ﻭﺧﻠﻴﻂ ﻛﻠﻮﺭﻳﺪ ﺍﻟﺼﻮﺩﻳﻮﻡ ﻭﺍﻟﻤﺎء. ﺑﺎﻹﺿﺎﻓﺔ ﺇﻟﻰ ﺫﻟﻚ، ﺃﺩﺕ ﺍﻟﻘﻴﺎﺳﺎﺕ ﺍﻟﺘﺠﺮﻳﺒﻴﺔ ﻟﻄﻴﻒ ﺍﻻﻧﺘﻘﺎﻝ ﺍﻟﺼﻮﺗﻲ

ﺇﻟﻰ ﺗﻄﻮﻳﺮ ﻣﻨﺤﻨﻴﺎﺕ ﺍﻟﻤﻌﺎﻳﺮﺓ ﻟﻜﻞ ﻣﺰﻳﺞ، ﻣﻤﺎ ﻣﻜﻦ ﻣﻦ ﺗﺤﺪﻳﺪ ﺍﻟﺘﺮﻛﻴﺰ. ﻛﻤﺎ ﺑﻴﻨﺖ ﻫﺬﻩ ﺍﻟﺪﺭﺍﺳﺔ ﺃﺩﺍء ﺍﻟﻤﺴﺘﺸﻌﺮ ﻣﻦ ﺧﻼﻝ ﻗﻴﺎﺳﺎﺕ

ﺍﻟﺤﺴﺎﺳﻴﺔ ﻭﻋﺎﻣﻞ ﺍﻟﺠﻮﺩﺓ .(Q-factor) ﺍﻟﻘﺴﻢ ﺍﻟﺜﺎﻟﺚ ﺗﻀﻤﻦ ﺇﻧﺸﺎء ﻭﺍﺟﻬﺔ ﻣﺴﺘﺨﺪﻡ ﺭﺳﻮﻣﻴﺔ (GUI) ﻟﺪﻋﻢ ﺟﻤﻊ ﺍﻟﺒﻴﺎﻧﺎﺕ ﻭﺗﺤﻠﻴﻠﻬﺎ

ﻓﻲ ﺍﻟﻮﻗﺖ ﺍﻟﻔﻌﻠﻲ. ﺧﻀﻌﺖ ﺍﻟﻮﺍﺟﻬﺔ ﻟﻼﺧﺘﺒﺎﺭ ﺍﻟﺬﻱ ﺃﻛﺪ ﺃﺩﺍﺋﻬﺎ ﺍﻟﺘﺸﻐﻴﻠﻲ ﺍﻟﻤﻮﺛﻮﻕ .

ii	Preface

This master’s thesis was conducted in the context of the master’s program in Physics of Mate- rials, taught by the physics department of the Setif University. The research was carried out within the Research Unit in Optics and Photonics (UROP) at the CDTA (Centre de Développement des Technologies Avancées). The research presented in this thesis was carried out between February 2, 2025, and May 25, 2025, at the Photonic Crystal Lab, affiliated with UROP-CDTA. This project was conducted under the supervisor Dr. KANOUNI Fares, a Senior Researcher Class A and team leader at UROP.

# iii	Acknowledgements

First and foremost, I would like to thank my master’s thesis supervisor, Dr. KANOUNI Fares, Senior Researcher (Class A) at the Research Unit in Optics and Photonics, affiliated with the Centre for Advanced Technology Development (CDTA), for his unwavering support.I am deeply grateful for his patience, motivation, and valuable guidance throughout the entire process.

I also extend my sincere gratitude to Dr. ARAB Fahima for her expert guidance and support, which greatly assisted me during the research and writing of this thesis.

My heartfelt thanks go to CHETTOUH A. Nidhal and Dr. Youcef Bougherira, director of UROP who provided an exceptional research environment and continuous support during my studies. Special thanks are due to all CDTA members, especially Dr. GUITOUME Djamel, Dr. RAHMANI Mahdi, and MOSBAH Daamouche, for their excellence in the fields of chemistry and mechanics. Their unconditional help and expertise contributed significantly to the smooth progress of this project.

It must also be acknowledged that the foundation of this research lies in the longstanding tradition of excellence at CDTA. I am grateful to the faculty and staff of the Physics Department at the University of Ferhat Abbas Sétif for providing a stimulating academic environment and access to essential resources. Special thanks go to the Chemistry Department for providing key materials used in chemical applications during the development of this thesis.

Finally, I owe my deepest personal gratitude to my parents and siblings for their steadfast belief in me throughout the challenges of this journey. Their encouragement was a constant source of motivation. This thesis is the result of collective efforts, and I am sincerely appreciative of everyone who contributed to its completion.

Thank you.

iv	Contents

Abstract	i

Preface	ii

Acknowledgements	iii

Contents	iv

Introduction	1

CHAPTER I: PHONONIC CRYSTALS FOR LIQUID SENSING APPLICATIONS	2

  - INTRODUCTION	3

  - ACOUSTIC WAVES	4

    - Fundamentals of Acoustic Waves	4

    - Types of Waves Based on Propagation Mode	5

      - Surface acoustic waves (SAW)	5

      - Bulk acoustic wave (BAW)	7

  - PHONONIC CRYSTALS: PRINCIPLES AND CLASSIFICATIONS	7

    - Fundamental concepts	7

      - Bragg Scattering Mechanism	8

      - Local Resonance Mechanism	9

    - Different types of phononic crystals	9

  - TECHNOLOGICAL APPLICATIONS OF PHONONIC CRYSTALS	10

    - Acoustic Filtering and wave-guiding	11

    - Acoustic absorbers devices	12

    - Sensing Applications	12

  - PHONONIC-FLUIDIC SENSORS	14

    - Working principal of the phononic-fluidic sensors	14

    - Literature review	15

    - Phononic crystal for Liquid characterization	16

      - One dimensional phononic for liquid detection	16

      - 2D Phononic crystal for liquid characterization	19

      - Printed 3D phononic crystal for liquid characterization	21

  - CONCLUSION	23

CHAPTER II: COMSOL-BASED NUMIRICAL ANALYSIS OF A PHONONIC- FLUIDIC SENSOR	24

  - INTRODUCTION	25

  - NUMERICAL ANALYSIS OF PHONONIC CRYSTAL	25

    - Geometry construction	26

    - Solid-mechanic and Bloch’s theorem	27

    - Pressure acoustic module for phononic-fluidic structure	28

    - Mesh condition	30

  - STUDY OF TRANSMISSION SPECTRUM AND BAND DIAGRAMS	30

    - Phononic crystal without liquid defect	30

    - Phononic crystal with distilled water defect Mode	33

      - Point defect modes	33

      - Line defect modes	34

  - PHONONIC CRYSTAL SENSOR FOR LIQUID MIXTURE ANALYSIS	36

    - Acetone–Water Mixture Characterization	36

      - Acoustic transmission	36

      - Calibration curve	37

      - Quality factor (Q-factor), Sensitivity, and Figure of Merit (FoM)	39

    - Nickel chloride -Water Mixture Characterization	41

      - Acoustic transmission	41

      - Calibration curve	42

      - Quality factor (Q-factor), Sensitivity and Figure of Merit (FoM)	42

    - Sodium chloride (NaCl)–Water Mixture Characterization	43

      - Acoustic transmission	43

      - Calibration curve	44

      - Quality factor (Q-factor), Sensitivity and Figure of Merit (FoM)	45

  - CONCLUSION	45

CHAPTER III: EXPRIMENTAL INVESTIGATION OF PHONONIC CRYSTALS FOR LIQUID SENSING APPLICATIONS	46

  - INTRODUCTION	47

  - PHONONIC CRYSTAL DESIGN AND FABRICATION	47

  - EXPERIMENTAL ANALYSIS OF THE PHONONIC BAND GAP	48

    - Phononic Band Gap without Liquid	48

    - Phononic Crystal Sensor Filled with Distilled Water	49

  - SAMPLES PREPARATION AND MEASUREMENT STEPS	50

    - Samples Preparation	51

    - Measurement steps	53

  - PHONONIC CRYSTAL SENSOR FOR LIQUID CHARACTERIZATION	55

    - Acetone–Water Mixture Characterization	55

      - Transmission measurement	55

      - Acetone concentration measurement and calibration curve	56

      - Measurement of Quality factor, Sensitivity and Figure of merit	58

    - Nickel Chloride –Water Mixture Characterization	58

      - Transmission measurement	58

      - Nickel Chloride concentration measurement and calibration curve	59

      - Measurement of Quality factor, Sensitivity and Figure of merit	60

      - Comparison with Other Technique: Electrical Conductivity Measurement	61

    - Sodium Chloride (NaCl)–Water Mixture Characterization	63

      - Transmission measurement:	63

      - Concentration measurement and calibration curve:	63

      - Measurement of Quality factors, Sensitivity and Figure of Merit	65

  - CONCLUSION	65

CHAPTER IV: DEVELOPMENT OF A GRAPHICAL USER INTERFACE GUI FOR THE PHONONIC-FLUIDIC SENSOR	66

  - INTRODUCTION	67

  - GRAPHICAL USER INTERFACE (GUI)	67

  - GRAPHICAL USER INTERFACE FOR PHONONIC CRYSTAL SENSOR	67

    - Physical Architecture Diagram	67

    - MATLAB Program	69

  - A GUI IMPLEMENTATION FOR REAL-TIME ACETONE MONITORING  70

    - Calibration: Upload the type of the liquid	70

    - Connecting to VNA	70

    - Results analysis	72

- 5. CONCLUSION	74

IV.6. ANNEX	75

OVERALL CONCLUSION	80

ACQUIRED SKILLS	82

REFERENCES	84

# INTRODUCTION

Phononic crystals (PnCs) represent a class of artificially structured materials, designed to con- trol, manipulate, and direct the propagation of acoustic waves through periodic arrangements of elastic materials with different acoustic properties [1-3]. The distinct feature of phononic crystals is their ability to exhibit phononic bandgap, a frequency ranges within which wave propagation is prohibited [4].

Phononic crystals can be classified based on their dimensionality into one-dimensional (1D), two-dimensional (2D), and three-dimensional (3D) structures [5-8]. 1D phononic crystals have periodicity along a single axis, are typically formed by alternating layers of materials, and are used in simple acoustic filters and reflectors. 2D phononic crystals feature periodic patterns in a plane—such as arrays of holes or rods—and are widely applied in sensors, waveguides, and sound isolation devices. 3D phononic crystals exhibit periodicity in all three spatial directions, offering complete control over acoustic wave propagation and enabling full band gaps, making them ideal for advanced sensing and vibration control applications. These specific properties make them highly suitable for applications in sound insulation, frequency filtering, and sensing technologies[9-11].

One of the challenging applications is their exploitation for sensing purposes, for example, for monitoring, controlling and measuring volumetric properties of complex liquids. For this purpose, Lucklum and Li proposed a phononic-fluidic cavity sensor [12]. In contrast with classical resonant sensors [13-16], e.g. quartz crystal microbalance (QCM), surface acoustic wave (SAW), and flexu- ral wave plate (FWP), the phononic-fluidic is able to measure bulk properties of liquids, not surface ones.

Moreover, accurately determining volumetric properties such as density and sound velocity is crucial for the precise characterization of complex mixtures. In this context, we conduct a research project on measuring in real-time the bulk properties of complex liquids such as acetone–water, nickel chloride–water and sodium chloride–water.

This dissertation is structured into four main chapters. Chapter I introduces the fundamental concepts of acoustic waves and phononic crystals, their classifications, and their technological ap- plications, with a particular focus on liquid sensing using phononic-fluidic structures. Chapter II presents a detailed numerical study of a phononic-fluidic sensor using COMSOL Multiphysics, in- cluding geometry design, simulation of wave propagation, and analysis of transmission spectra for various liquid mixtures. Chapter III focuses on the experimental design, fabrication, and testing of 2D phononic crystal sensors, with validation through acoustic transmission measurements and per- formance evaluation for different liquid mixtures. Finally, Chapter IV describes the development of a custom graphical user interface (GUI) to facilitate real-time control, data acquisition, and analysis of the phononic sensor, enhancing its usability for practical sensing applications.

1