وزارة التعليم العالي والبحث العلمي

| Setif 1 University Ferhat Abbas Faculty of Sciences Department of Physics |  | # جامعـــةسطيف 1 فرحـــــــات عباس # كـليــــة العـلـــــــــــــوم # قســـــــــــم الفيزياء |
| --- | --- | --- |

MASTER THESIS

Field: Matter Science

Branch: Physics

Specialty: Radiation Physics

Title

The calculation of the radiological properties and Response of

a Polymer Gel Dosimeter Using UV-Visible Spectrophotometer

| Presented by Sedira Fairouz Yahia Salsabil |  | Directed by Pr . Nadjla Bourbia |
| --- | --- | --- |

Examination jury :

| President | O.Boukhenfouf MCA | University Ferhat Abbas sétif 1 |
| --- | --- | --- |
| Supervisor | N. Bourbia MCA | University les fréres Mentouri Constantine 1 |
| Examiner | M.Houas MCA | University Ferhat Abbas sétif1 |

2024 / 2025

Table of contents

INTRODUCTION	1

Chapter I : Radiation Matter Interaction

I.1.Introduction	2

I.1.1.Radiation	2

I.1.2.Living matter	2

I.1.3.Classification of radiation	3

I.1.4.The different ionizing radiation	4

I.2.Interction of particles with matter	4

I.3.Interaction of charged particles		5

I.3.1.Electro_matter interaction	5

I.3.1.1.Exictation and Ionization phenomenon	6

I.3.1.2.Braking radiation (Bremsstrahlung)	7

I.4.Interaction of heavy charged particles with matter	8

I.5.Characteristic Quantities of these interactions	8

I.5.1.Linear Energy Transfer (LET)	8

I.5.2.The total mass stopping power	9

I.6.Interactions of electromagnetic radiation with matter	10

I.6.1.photoelectric effect	10

I.6.2.Compton Scattering 	11

I.6.3.Pair production	12

I.6.4.Relative importance of the three photon-matter interaction processes	13

I.6.5.The attenuation law of electromagnetic radiation	14

I.6.6.Cross Section	15

I.6.7.Effective atomic number	15

ChapterII: Dosimetry

II.1.Introduction	16

II.2.Dosimetric Quantities and Units	16

II.2.1.Exposure	16

II.2.2.Kerma (Kinetic Energy Released per Unit Mass)	16

II.2.3.The absorbed dose	17

II.2.4.The relation Dose_Kerma		17

II.2.5.Absorbed dose rate	17

II.3.Dosimetric Gels	18

II.3.1.Fricke gels	18

II.3.2.Polymer gels	ss20

II.3.3.Advantages of polymer gel dosimetry	20

II.4.Radiation-Induced Polymerization	20

II.5.Thechniques of gel dosimetry evaluation	21

II.5.1.Magnetic resonance imaging	21

II.5.2.X-ray Computed Tomography (CT)	22

II.5.3.Optical Tomography	22

II.6.Multi-Sample Calibration	23

II.7.MAGIC normoxic gel	24

ChapterIII: Equivalent tissue

III.1.Introduction	26

III.2.Material and method	26

a.MAGIC Gel components	26

b.Mass fraction	27

c.Density measurements	27

d.The effective atomic number	27

e.Number of electrons per gram	27

f.Electronic density	28

g.Photon attenuation and absorption	28

i.The total mass stopping power	28

III.3.Results and discussion	28

a.mass fraction	28

b.The number of electrons per gram and electronic density	30

c.The effective atomic number	30

d.Total mass attenuation coefficient	31

e.Interaction probability fractions	32

f.Total stopping power	32

g.Absorption coefficient	33

III.4.Conclusion	34

ChapterIV:Applications

IV.1.Introduction	35

IV.2.Materials and Methods	35

IV.3.MAGIC gel irradiation procedure	36

IV.4.Characterization of the Gel using UV/Vis Spectrophotometry	37

IV.5.Results and discussion	38

IV.6.Conclusion	40

CONCLUSION	41

LIST OF FIGURES :

Figure I.1.ionization phenomenon	6

Figure I.2.Exitation phenomenon	7

Figure I.3.Braking radiation	7

Figure I.4.Variation of LET in tissues as a function of electron energy	9

Figure I.5.Main photon-matter interaction processes	10

Figure I.6.Schematic of photoelectric absorption processes	11

Figure I.7.Schematic of compton Scattering	12

Figure I.8.Schematic of pair production	13

Figure I.9.The relative importance of the three effects as a function of the energy of the incident photon and the atomic number of the medium	13

Figure I.10.The attenuation of a photon beam in matter	14

Figure III.1.Variation of the effective atomic number (Zeff) of MAGIC gel ,water and soft tissue	30

Figure III.2.Ratios between the mass attenuation coefficient ρµ(cm2g) of water or soft tissue and those of MAGIC gel for photons from 0.01MeV to 10MeV	31

Figure III.3.Calculated interaction probability fractions of MAGIC gel and water	32

Figure III.4.Total stopping power of MAGIC gel and water	33

Figure III.5.Absorption coefficient of MAGIC gel and water	33

FigureIV.1.Some materials and tools used in the preparation of MAGIC gel	35

FigureIV.2.Fabrication of MAGIC gel	36

FigureIV.3.X-ray photons and CLINAC 2100 SC linear accelerator	37

FigureIV.4. double-beam spectrophotometer  (UV/Vis)	37

FigureIV.5. MAGIC gel samples after irradiation at 0,2,4,6,8Gy with 6MeV	38

FigureIV.6.Optical absorbance curves as a function of wavelengths.	38

FigureIV.7.The optical absorbance of the studied MAGIC gel versus the absorbed dose was measured at three wavelengths	39

LIST OF TABLES :

Table III.1.The MAGIC gel components	26

Table III.2.Mass fraction of MAGIC gel ,water and soft tissue	29

Table III.3.Number of electrons per gram and electronic density of MAGIC gel ,water and soft tissue	30

LIST OF ABBREVIATIONS :

BANG: Bis Acrylamide Nitrogen and Gelatin

ICRU: International Commission on Radiation Units and Measurements

PAG: Poly Acrylamide Gel

MAGIC : Methacrylic and Ascorbic acids in Gelatin Initiated by Copper

HQ : HydroQuinone

CT : Computed Tomography

RNA : Ribonucleic Acid

TPS :Treatement Planing System

MRI : Magnetic Resonance Imagning

3D : Three Dimensional

MAA : Methacrylic Acid

AA : Ascorbic Acid

Abstract

Polymer gel dosimeters are among the most commonly employed techniques for radiation measurement in radiotherapy due to their three-dimensional (3D) dose recording capabilities, high spatial resolution, and flexibility in shaping and sizing the detector to meet specific requirements. Currently, the primary imaging modalities used to analyze these dosimeters post-irradiation include Magnetic Resonance Imaging (MRI), ultrasound, and X-ray Computed Tomography (CT). In this study, we prepared the MAGIC gel dosimeter and investigated its water equivalence, as well as key physical interaction parameters such as mass fraction (𝑤𝑎), effective atomic number (𝑍𝑒𝑓𝑓), and attenuation coefficients. These characteristics were compared with those of water and biological tissues, particularly soft tissue. To carry out this analysis, we employed the XCOM database to calculate photon attenuation coefficients over an energy range from 1 keV to 10 MeV, PHY-X/ Zextra software to determine the effective atomic number, NIST ESTAR to evaluate the stopping power, and Phys-X to compute the absorption coefficient.

The MAGIC gel was prepared as part of this study, and irradiation was performed using a CLINAC 2100 SC linear accelerator. The gel’s response to radiation will be assessed post-irradiation using a spectrophotometer.

Keywords: MAGIC gel, Polymer gel dosimetry, XCOM database, Attenuation coefficient, Tissue equivalence, Stopping power, Water equivalence, Irradiation.

Résumé

Les dosimètres en gel polymère sont parmi les techniques les plus couramment utilisées pour la mesure des rayonnements en radiothérapie, en raison de leur capacité d’enregistrement tridimensionnel (3D) de la dose, de leur haute résolution spatiale et de leur flexibilité en termes de forme et de taille, permettant d’adapter le détecteur à des exigences spécifiques. Actuellement, les principales modalités d’imagerie utilisées pour analyser ces dosimètres après irradiation incluent l’Imagerie par Résonance Magnétique (IRM), l’échographie et la tomodensitométrie (CT) aux rayons X.

Dans cette étude, nous avons préparé le dosimètre en gel MAGIC et avons étudié son équivalence à l’eau, ainsi que des paramètres clés d’interaction physique tels que la fraction massique (𝑤𝑎), le numéro atomique effectif (𝑍𝑒𝑓𝑓) et les coefficients d’atténuation. Ces caractéristiques ont été comparées à celles de l’eau et des tissus biologiques, en particulier les tissus mous.

Pour réaliser cette analyse, nous avons utilisé la base de données XCOM pour calculer les coefficients d’atténuation des photons sur une gamme d’énergies allant de 1 keV à 10 MeV, le logiciel PHY-X/ Zextra pour déterminer le numéro atomique effectif, le NIST ESTAR pour évaluer le pouvoir d’arrêt, et Phys-X pour calculer le coefficient d’absorption.

Le gel MAGIC a été préparé dans le cadre de cette étude, et l’irradiation a été réalisée à l’aide d’un accélérateur linéaire CLINAC 2100 SC. L’analyse post-irradiation sera effectuée à l’aide d’un spectrophotomètre afin d’évaluer la réponse du gel au rayonnement.

Mots-clés :Gel MAGIC, Dosimétrie par gel polymère, Base de données XCOM, Coefficient d’atténuation, Équivalence tissulaire, Pouvoir d’arrêt, Équivalence à l’eau, Irradiation.

الملخص

تعد المقاييس الإشعاعية المصنوعة من الجل البوليمري من أكثر التقنيات استخدامًا في مجال قياس الجرعات الإشعاعية في العلاج الإشعاعي، وذلك لما تتميز به من قدرة على تسجيل التوزيع ثلاثي الأبعاد(3D)للجرعة، وبدقة مكانية عالية، بالإضافة إلى مرونتها في التشكيل والحجم، مما يسمح بتكييف الكاشف وفقًا لمتطلبات القياس المختلفة.تُستخدم حاليًا عدة تقنيات تصوير رئيسية لتحليل هذه المقاييس بعد التشعيع، من بينها التصوير بالرنين المغناطيسي (MRI)، والتصوير بالموجات فوق الصوتية، والتصوير المقطعي المحوسب بالأشعة السينية (CT).في هذه الدراسة، قمنا بتحضير مقياس إشعاعي باستخدام جل MAGIC، ودرسنا مدى مكافأته للماء، بالإضافة إلى عدد من معلمات التفاعل الفيزيائي الأساسية، مثل الكسر الكتلي (𝑤𝑎)، والعدد الذري الفعّال (𝑍𝑒𝑓𝑓)، ومعاملات التوهين. وقد تمّت مقارنة هذه الخصائص مع تلك الخاصة بالماء وبعض الأنسجة البيولوجية، وبشكل خاص الأنسجة الرخوة.لإجراء هذا التحليل، تم استخدام قاعدة بيانات XCOM لحساب معاملات توهين الفوتونات ضمن نطاق طاقة يتراوح من 1 keV إلى 10 MeV، كما تم استخدام برنامج PHY-X/Zextra لتحديد العدد الذري الفعّال، وأداة NIST ESTAR لحساب قدرة الإيقاف، بالإضافة إلى برنامج Phys-X لحساب معامل الامتصاص.

تم تحضير هلام MAGIC في إطار هذه الدراسة، وتمت عملية التشعيع باستخدام معجّل خطي من نوع CLINAC 2100 SC، وسيتم تقييم استجابة الهلام للإشعاع بعد التشعيع باستخدام مقياس الطيف (Spectrophotometer).

الكلمات المفتاحية:جل MAGIC، القياس الإشعاعي بالجل البوليمري، قاعدة بيانات XCOM، معامل التوهين، مكافأة الأنسجة، قدرة الإيقاف، مكافأة الماء، التشعيع